2.5.5/Ionsaco Gheorghe

Part 1

Step 2

How many Fast Ethernet interfaces does the switch have? – 24

How many Gigabit Ethernet interfaces does the switch have? – 2

What is the range of values shown for the vty lines? 0 -15

Which command will display the current contents of non-volatile random-access memory (NVRAM)? ---

--- show startup-configuration

Why does the switch respond with “startup-config is not present?”

It displays this message because the configuration file was not saved to NVRAM .Currently it is only located RAM.

Part 2

Step 2

Why is the login command required?  ---

--- In order for the password checking process to work, it requires both the login and password commands.

Step 7

What is displayed for the enable secret password? --- $1$mERr$ILwq/b7kc.7X/ejA4Aosn0

Why is the enable secret password displayed differently from what we configured? --- The enable secret is shown in encrypted form, whereas the enable password is in plain text.

Step 8

If you configure any more passwords on the switch, will they be displayed in the configuration file as plain text or in encrypted form? --- Explain. The service password-encryption command encrypts all current and future passwords.

Part 3

Step 1

When will this banner be displayed? --- The message will be displayed when someone enters the switch through the console port.

Why should every switch have a MOTD banner? --- Every switch should have a banner to warn unauthorized users that access is prohibited but can also be used for sending messages to network personnel/technicians (such as impending system shutdowns or who to contact for access)

Part 4

Step 1

What is the shortest, abbreviated version of the copy running-config startup-config command? –

--- cop r s

Which command will display the contents of NVRAM? --- show startup-config

Are all the changes that were entered recorded in the file? ---

Yes, it is the same as the running configuration.